# Chapter 3

## BIOS Setup

### Introduction

The BIOS setup utility stores information about your computer such as the date and time, the kind of hardware you have installed, and so on. Your computer uses this information to initialize all the components at boot up time, and make sure that everything runs smoothly.

If the information in the setup utility is incorrect, it may cause your system to malfunction. It can even stop your computer from booting properly. If this happens, you can use the clear CMOS jumper to clear the CMOS memory area that is used to store the setup information, or you can hold down the End key while you reboot your computer, Holding down the End key also clears the setup information.

You can run the setup utility and manually make changes to the setup utility. You might need to do this to configure some of the hardware that you add to the mainboard, such as the CPU, the memory, disk drive, etc.

### Running the Setup Utility

Each time your computer starts, before the operating system is booted, a message appears on the screen that prompts “Hit <DEL> if you want to run SETUP”. When you see this message, press the Delete key and the Mainmenu page of the setup utility appears on your monitor.

You can use the cursor arrow keys to highlight any of the options on the Mainmenu page. Press Enter to select the highlighted option. To leave the setup utility, press the Escape key. Hold down the Shift key and press F2 to cycle through the optional color schemes of the setup utility.

Some of the options on the Mainmenu page lead to tables of items with installed values. In these pages, use the cursor arrow keys to highlight the items, and then use the PgUp and PgDn keys to cycle through the alternate values for each of the items. Other options on the Mainmenu page lead to dialog boxes which require you to answer Yes or No by hitting the Y or N keys.

If you have already made changes to the setup utility, press F10 to save those changes and exit the utility. Press F5 to reset the changes to the original values. Press F6 to install the setup utility with a set of default values. Press F7 to install the setup utility with a set of high-performance values.

### Standard CMOS Setup Page

Use this page to set basic information such as the date and time, the IDE devices, and the diskette drives. If you press the F3 key, the system will automatically detect and configure the hard disks on the IDE channels.

| Date & Time | Use these items to install your system with the correct date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | Use these items to configure devices on the primary and secondary IDE channels. To configure a hard disk drive, choose Auto. If the Auto setting fails to find a hard disk drive, set it to User, and then fill in the hard disk characteristics (Size, Cyls, etc.) manually. If you have a CD-ROM drive, select the setting CDROM. If you have an ATAPI device with removable media (e.g. a ZIP drive or an LS-120) select ARMD. |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. |

### Advanced Setup Page

Use this page to set more advanced information about your system. Take some care with this page. Making changes can affect the operation of your computer.

| Share Memory Size | This item lets you allocate a portion of the main memory for use by the onboard VGA system. |
| --- | --- |
| Trend ChipAway Virus | This mainboard has built-in virus protection in the firmware. Use this item to enable or disable the built-in virus protection. |
| 1st Boot Device 2nd Boot Device | Use these two items to determine the order and priority that your computer follows to load an operating system at start-up time. |
| Try Other Boot Device | If you enable this item, the system will also search for other boot devices if it fails to find an operating system from the first two locations. |
| S.M.A.R.T. for Hard Disks | Enable this item if your hard disk(s) supports SMART (Self-Monitoring, Analysis and Reporting Technology). |
| BootUp Num-Lock | This items determines if the Num Lock key is active or inactive at system start-up time. |
| Floppy Drive Swap | If you have two diskette drives installed and you enable this item, drive A becomes drive B and drive B becomes drive A. |
| Floppy Drive Seek | If you enable this item, your system will check the diskette drives at start up time. Disable this item unless you are using an old 360K diskette drive. |
| Password Check | If you have installed a password on your system, use this item to determine if the password is required to enter the setup utility (Setup) or required at start-up time and to enter the setup utility (Always). |
| Boot to OS/2 Over 64MB | Enable this item if you are booting the OS/2 operating system and you have more than 64MB of memory installed. |
| Internal Cache | Leave this item enabled since all the processors that can be installed on this board have internal cache memory. |
| System BIOS Cacheable | If you enable this item, a segment of the system BIOS will be cached to main memory for faster execution. |
| SDRAM Speed | This item determines the speed of the installed SDRAM main memory. We recommend that you leave this item at the default value. |
| Refresh Queue Depth | This item determines the operation of the memory refresh timing. We recommend that you leave this item at the default value. |
| SDRAM CAS Latency | This item determines the operation of the main memory CAS (column address strobe). We recommend that you leave this item at the default value. |
| Graphic Win Size | This item determines the operation of the onboard graphics adapter. We recommend that you leave this item at the default value. |

### Power Management Setup Page

This page sets some of the parameters for the system power management operation.

| Power Management/APM | Use this item to enable or disable the power management routines. If you enable the power management, you can use the items below to set the power management operation. |
| --- | --- |
| Standby Time Out (Minute) | This sets the timeout for standby mode in minutes. If the time selected passes without any system activity, the computer will enter the power-saving standby mode. |
| Suspend Time Out (Minute) | This sets the timeout for suspend mode in minutes. If the time selected passes without any system activity, the computer will enter the power-saving suspend mode. |
| Keyboard Power On | Your system can enter a software power down. If you enable this item, you can resume the system by pressing hot keys on the keyboard. You must be using an ATX power supply and enable jumper JP1 in order to use this feature. |
| Lan Card Power On | Your system can enter a software power down. If you enable this item, the system can automatically resume if there is traffic on the network adapter. You must have an ATX power supply in order to use this feature. |
| Ring On Power On | Your system can enter a software power down. If you enable this item, the system can automatically resume if there is an incoming call on the built-in Fax/Modem. You must have an ATX power supply in order to use this feature. |
| RTC Alarm Power On | Your system can enter a software power down. If you enable this item, the system can automatically resume at a fixed time on the system’s RTC (realtime clock). Use the items below to set the date and time of the wake-up alarm. You must have an ATX power supply in order to use this feature. |

### PCI / Plug and Play Setup Page

This page sets some of the parameters for devices installed on the system PCI bus, and devices that use the system plug and play capability.

| Plug and Play Aware O/S | Enable this item if you are using an O/S that supports Plug and Play such as Windows 95 or 98. |
| --- | --- |
| Primary Graphics Adapter | Use this item to define if your primary graphics adapter is installed in a PCI slot or on an AGP bus. |
| PCI VGA Palette Snoop | When this item is enabled, multiple VGA devices operating on different buses can handle data from the CPU on each set of palette registers on every video device. |
| Allocate IRQ to PCI VGA | If this item is enabled, an IRQ will be assigned to the PCI VGA graphics system. We recommend that you set this value to No. |
| DMA Reserved for ISA | If you set it to a DMA, the channel will be reserved for an installed non-PnP ISA or EISA expansion card. |
| IRQ Reserved for ISA | If you set it to an IRQ, the IRQ line will be reserved for an installed non-PnP ISA or EISA expansion card. |
| Reserved Memory Size | This item lets you reserve a block of memory for any device that requires it. |
| Reserved Memory Address | This item lets you set the address for any block of memory that has been reserved. |

### Load Optimal Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the setup utility is loaded with a set of optimal default values. The optimal default values are not very demanding and they should allow your system to function with most kinds of hardware and memory chips.

### Load Best Performance Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the setup utility is loaded with a set of best-performance default values. The optimal default values are quite demanding and your system might not function properly if you are using slower memory chips or other kinds of low-performance components.

### Features Setup Page

This page sets some of the parameters for peripheral devices installed on the system.

| Onboard FDC | Use this item to enable or disable the onboard floppy disk drive interface. |
| --- | --- |
| Onboard Serial Port1 | Use this item to enable or disable the onboard serial port COM1, and to assign a port address |
| Onboard IR Port | Use this item to define the protocol for an infrared port if you have installed an optional IR port. |
| IR Duplex | Use this item to define if the optional infrared port is full-duplex or half-duplex. |
| Onboard Parallel Port | Use this item to enable or disable the onboard parallel port LPT1, and to assign a port address |
| Parallel Port Mode | Use this item to determine the parallel port mode. You can select Normal, ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. |
| Parallel Port IRQ | Use this item to assign an IRQ to the parallel port. |
| Parallel Port DMA | Use this item to assign a DMA channel to the parallel port. |
| Onboard IDE | Use this item to enable or disable either of the two onboard IDE channels, Primary or Secondary. |
| Ultra DMA Support | Use this item to determine if you require Ultra DMA support for IDE devices on the primary or secondary IDE channels. |
| USB Function | Enable this item if you plan to use the USB ports that are integrated on this mainboard. |
| USB Function for DOS | Enable this item if you plan to use the USB ports that are integrated on this mainboard in the DOS environment. |

### CPU PnP Setup Page

This page lets you manually configure the mainboard for the CPU. The system will automatically detect the kind of CPU that you have installed and make the appropriate adjustments to the items on this page.

| CPU Speed | Use this item to set the internal clock speed of your CPU. |
| --- | --- |
| CPU Base Frequency | Use this item to set the system bus frequency that is required by the CPU. |
| CPU Multiple Factory | Use this item to set a multiplier for the system bus frequency. The multiplier x the system bus frequency must equal the internal clock speed of your CPU, e.g. 100 MHz (system bus) x 3.5 (muliplier) = 350 MHz (internal clock speed of installed CPU). |
| SDRAM Frequency | Use this item to set the bus frequency for the memory bus. |

### Hardware Monitor Page

This page sets some of the parameters for the hardware monitoring function of this mainboard.

| CPU Temperature | Use this item to set the threshold temperature for the CPU. The system will alert you if this temperature is exceeded. |
| --- | --- |
| CPU FAN Speed & Vcore | Use these items to set the parameters for the voltages that control the fans and the CPU core. If the values deviate by an amount greater than the values installed here, the hardware monitoring will alert you with a warning . |

### Change Password

If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than six letters or numbers. Press Enter after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press Enter after you have retyped it correctly. The password is required at boot time, or when the user enters the setup utility.

#### Change or Remove the Password

Highlight this item and type in the current password. At the next dialog box, type in the new password, or just press Enter to disable password protection.

### Exit

Highlight this item and press Enter to save the changes that you have made in the setup utility and exit the setup program. When the Save and Exit dialog box appears, press Y to save and exit, or press N to exit without saving.